Strategic Report:

Edge AI Deployment Architectures

Hardware Targets: RDK X5 (8GB RAM, 10 TOPS BPU) & Qualcomm QCS6490 (Low-power NPU)

Technical Evaluation Report

January 2026

# Table of Contents

Right-click the TOC and select "Update Field" to refresh page numbers

Executive Recommendation	3

Strategy A: The Modular Split Architecture	4

Strategy B: The Monolithic Fine-Tuned VLM	6

The Verdict: Context-Dependent Implementation	8

References	10

# Executive Recommendation

For the Pocket AI Guardian and Vision AI Standalone platforms, the Split Modular Approach (Strategy A) is superior for core operations, safety, and battery life. The Monolithic VLM Approach (Strategy B) is currently viable only for specific, user-initiated "Assistant" tasks on the wired RDK X5 due to memory and latency constraints.

# Strategy A: The Modular Split Architecture

Concept: Running specialized, small vision models (e.g., RF-DETR, YOLO) to extract structured data (JSON), fed into a separate small reasoning model (e.g., Phi-3, SmolLM2) or logic engine.

## Pros (Why this fits your hardware)

### Performance & Latency

- Safety-critical features (like the Driver Distraction project) require stable 30+ FPS. Specialized detectors like RF-DETR-Nano or YOLO11-Nano achieve high frame rates (detecting objects in <3ms) on edge devices.

- Qualcomm's NPU and the RDK X5's BPU (Brain Processing Unit) are optimized for INT8 quantized detection models. You can use V4L2 zero-copy pipelines to feed these efficient models directly.

### Memory Efficiency

- Detection models are tiny (<20MB). Even with a small LLM for reasoning, the total footprint stays well under the 8GB RAM limit of the RDK X5, leaving room for the OS and orchestration daemon.

- This is critical for the Qualcomm QCS6490, which targets ultra-low power (80-150mW) for always-on sensing.

### Logic & Counting Accuracy

- The "Package Count Tracker" project proved that counting is a logic/tracking problem, not just detection. A split architecture allows you to insert classical computer vision logic (physics validation, geometry) between detection and reasoning, which prevents hallucinations common in VLMs.

## Cons

- Context Blindness: The text model only "knows" what the vision model detects. If the detector misses a "puddle," the reasoning engine cannot warn the blind user about it, even if the LLM is smart.

- Complexity: Requires an orchestration daemon to manage data flow and "hot-swap" models based on context (e.g., switching from "Gym Mode" to "Navigation Mode").

# Strategy B: The Monolithic Fine-Tuned VLM

Concept: Running a single, multi-modal model (e.g., SmolVLM, LiaVA) that takes raw pixels and text prompts to generate descriptive answers.

## Pros

- Deep Reasoning: A VLM like SmolVLM (2.2B parameters) can answer complex questions about relationships in an image (e.g., "Is the ground uneven?" or "Describe the cooking process") without needing specific class labels pre-programmed.

- Simplicity: Eliminates the need for intermediate JSON schemas or complex tracking logic. You just feed the image and prompt to the model.

- Reduced Hallucination (with DPO): Recent techniques like Direct Preference Optimization (DPO) have successfully reduced VLM hallucinations (e.g., claiming ships exist when they don't).

## Cons (Why this fails for your specific constraints)

### Memory Ceiling

- SmolVLM requires approximately 5.02 GB of GPU RAM even when optimized.

- The RDK X5 has only 8GB total shared RAM. With 2GB reserved for the OS/Orchestration, running a 5GB model leaves almost zero headroom for audio processing, system overhead, or OS spikes, leading to crashes or OOM (Out of Memory) kills.

### Unacceptable Latency

- VLMs are generative. While SmolVLM is fast for its class, it is still significantly slower than detection models. The "Vision AI" documentation targets ~5 FPS for VLM tasks, which is too slow for real-time safety warnings (falling, collisions) that require <600ms latency.

### Thermal Throttling

- Continuous VLM inference pushes power draw to 8-15W on the RDK X5. This is unacceptable for the wearable Pocket Guardian (Qualcomm QCS), which requires passive cooling and skin-safe temperatures.

# The Verdict: Context-Dependent Implementation

You should adopt the Layered AI Architecture proposed in your ideology document.

## 1. Run Strategy A (Split Models) for "Always-On" Safety

- Device: Pocket Guardian (QCS6490) & RDK X5 (Guardian Mode).

- Vision: RF-DETR or YOLO11 (Fine-tuned on custom datasets for specific objects like cars/obstacles) running on the NPU.

- Pose: YOLOv8-pose for activity tracking.

- Reasoning: A lightweight logic engine or a highly quantized small Language Model (SLM) like SmolLM2-1.7B only activated when an event is detected.

Why: This guarantees the 30 FPS required for safety and fits within the thermal envelope.

## 2. Run Strategy B (Monolithic VLM) for "User-Initiated" Tasks

- Device: RDK X5 (Stationary/Desktop Mode only).

- Model: SmolVLM (Quantized to 4-bit/8-bit).

- Use Case: The "Productivity Hub" or "Assist Mode" where a user explicitly asks, "Read this document" or "What is in front of me?"

Why: In these moments, 2-3 seconds of latency is acceptable, and the depth of understanding provided by SmolVLM (OCR, dense captioning) outperforms simple detectors.

## Implementation Path

- Start with Strategy A: Train RF-DETR on your "Car Maker" or "Obstacle" dataset. Use Outlines to force structured output from your logic engine to ensure reliability.

- Add Strategy B later: Once the hardware platform is stable, deploy SmolVLM as a "plugin" that is loaded into memory only when the user specifically requests detailed scene analysis, unloading it immediately after to free up RAM.

# References

The following sources were consulted to provide technical specifications, performance benchmarks, and recent research findings referenced throughout this report.

## Hardware Specifications & Benchmarks

[1] RDK X5 Review: AI Board for Robotics
https://medium.com/@zlodeibaal/rdk-x5-review-ai-board-for-robotics-017454547bc8
Comprehensive performance benchmarks including ResNet-18 (~449 FPS), YOLOv5 (33.99ms forward pass), and EfficientNet results on RDK X5 hardware.

[2] D-Robotics RDK X5 Official Documentation
https://doc.switch-science.com/media/files/1bc25b40-4713-42e4-826a-f95f3b84a8c6.pdf
Official specifications: 10 TOPS BPU, Octa-core Cortex-A55 @1.5GHz, 8GB LPDDR4 RAM, MIPI CSI/DSI interfaces, CAN FD support.

[3] Waveshare RDK X5 Product Page
https://www.waveshare.com/rdk-x5-md.htm
Detailed hardware specifications including Wi-Fi 6, Bluetooth 5.4, Gigabit Ethernet, USB 3.0/2.0 ports, and power requirements (5V/5A).

[4] Thundercomm RUBIK Pi 3 (QCS6490)
https://www.thundercomm.com/product/rubik-pi/
QCS6490-based platform with 8GB LPDDR4x, 128GB UFS, 12 TOPS AI performance. Power consumption: ~8W under full CPU/GPU load, 4W CPU-only, 5mA sleep mode.

[5] Radxa Dragon Q6A featuring Qualcomm QCS6490
https://linuxgizmos.com/radxa-rolls-out-dragon-q6a-featuring-qualcomm-qcs6490-12-tops-npu-and-6th-gen-ai-engine/
QCS6490 specifications: Kryo 670 CPU (1x Prime @2.7GHz, 3x Gold @2.4GHz, 4x Silver @1.9GHz), Adreno 643 GPU, 6th-gen AI Engine with 12 TOPS.

[6] TRIA SM2S-QCS6490 Datasheet
https://embedded.avnet.com/wp-content/uploads/2024/03/MSC-SM2S-QCS6490.pdf
Industrial QCS6490 module: 7W typical power consumption, 2x Gigabit Ethernet, MIPI interfaces, extended temperature range (-30 to 85C).

## AI Model Performance & Deployment

[7] RF-DETR: Real-Time Object Detection with Speed and Accuracy
https://caasify.com/rf-detr-real-time-object-detection-with-speed-and-accuracy/
RF-DETR achieves 60+ mAP on COCO benchmark, first real-time model to exceed 60 mAP. Base variant: 29M parameters, optimized for edge deployment.

[8] RF-DETR vs YOLO: First 60 AP Real-Time Detector
https://medium.com/@aedelon/yolo-is-dead-welcome-rf-detr-the-transformer-that-just-shattered-the-60-map-barrier-e814475d9f8c
RF-DETR achieves 60.5 mAP at 25 FPS on NVIDIA T4. RF-DETR-S beats YOLO11-X with higher mAP (53.0 vs 51.2) while running faster (3.52ms vs 11.92ms latency).

[9] RF-DETR Object Detection vs YOLOv12 (Research Paper)
https://arxiv.org/html/2504.13099v1
Academic comparison showing RF-DETR's rapid convergence (plateauing under 10 epochs) and superior performance in agricultural detection scenarios with mAP@50 of 0.9464.

[10] YOLO11 vs YOLOv8: Model Comparison
https://www.labellerr.com/blog/yolo11-vs-yolov8-model-comparison/
YOLO11n latency benchmarks: ~16ms/frame (~61 FPS) for edge applications. Comparison of accuracy vs speed trade-offs across Nano, Small, Medium, and Large variants.

[11] YOLO11 Nano Deep Learning Model Analysis
https://www.emergentmind.com/topics/yolo11n-deep-learning-model
YOLO11n demonstrated best-in-class edge deployment: 4 FPS including actuation in weed spraying robots, sub-10MB model size, robust to small and occluded objects.

## Vision-Language Models & Memory Requirements

[12] SmolVLM: Redefining small and efficient multimodal models
https://openreview.net/forum?id=qMUbhGUFUb
SmolVLM-256M uses <1GB GPU memory during inference. SmolVLM-2.2B rivals state-of-the-art VLMs while consuming half the GPU memory (~5GB vs 13-16GB for Qwen2-VL).

[13] SmolVLM to SmolVLM2: Compact Models for Multi-Image VQA
https://pyimagesearch.com/2025/06/23/smolvlm-to-smolvlm2-compact-models-for-multi-image-vqa/
SmolVLM2 comes in 256M, 500M, and 2.2B parameter variants. The 2.2B model requires only ~5GB GPU memory for inference with image input, supporting both image and video understanding.

[14] Best Local Vision-Language Models for Offline AI
https://blog.roboflow.com/local-vision-language-models/
Comprehensive review of compact VLMs including SmolVLM2, InternVL, and other edge-deployable multimodal models with memory efficiency comparisons.

## Hallucination Reduction & Optimization Techniques

[15] Hallucination-Aware Direct Preference Optimization
https://www.emergentmind.com/topics/hallucination-aware-direct-preference-optimization-ha-dpo
HA-DPO extends Direct Preference Optimization by incorporating hallucination-specific heuristics, demonstrating significant reduction in hallucination rates across vision, audio, and text models.

[16] CLIP-DPO: Vision-Language Models as a Source of Preference
https://www.ecva.net/papers/eccv_2024/papers_ECCV/papers/09810.pdf
Novel approach using pre-trained CLIP models to rank generated outputs for DPO training, removing need for paid APIs or external LVLMs while fixing hallucinations.

[17] V-DPO: Mitigating Hallucination in Large Vision Language Models
https://aclanthology.org/2024.findings-emnlp.775.pdf
Vision-guided DPO enhances visual context learning at training time, demonstrating superior ability to understand nuances of visual context compared to baseline methods.

[18] Optimizing Transformer Models for Edge Deployment
https://emergingpub.com/index.php/em/article/download/79/65/255
Comprehensive study on quantization-aware training (QAT), pruning, and knowledge distillation for vision transformers. MobileViT with QAT reduced model size from 52MB to 29MB while maintaining 90%+ accuracy.

[19] Vision Transformers on the Edge: A Comprehensive Survey
https://arxiv.org/html/2503.02891v1
Survey covering model compression (pruning, quantization, knowledge distillation) and hardware-aware acceleration strategies for ViT deployment on edge devices including GPUs, FPGAs, and TPUs.

[20] Right Sizing Your Edge AI Hardware
https://www.onlogic.com/blog/right-sizing-your-edge-ai-hardware/
YOLO11 benchmarks on NVIDIA RTX 4000 Ada: 954 FPS (nano), 167 FPS (extra-large) with TensorRT optimization on 640x640 streams, demonstrating hardware capability requirements.

Edge AI Deployment Architectures

Technical Evaluation Report

January 2026